# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title:   Supply, Delivery, Installation and Commissioning of information communication and technology (ICT) and electronic equipment to the Business Regulatory Review Agency (BRRA), Zambia Development Agency (ZDA), Ministry of Green Economy and Environment (MGEE), Competition and Consumer Protection Commission (CCPC), and Patents and Companies Registration Agency (PACRA) 	p 1 /…

Publication reference: NAO/EDF/TABI/SUP/2026-01

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Lot 1:  Supply, Delivery, Installation and Commissioning of Laptops, Desktops, All-in-one Desktop Computers , UPSs, Adobe Creative Suite (CS), Microsoft Office Enterprise, Microsoft PowerBI, Office 2024 (volume License) and Windows Server licenses (Windows Server 2025 Datacenter), Servers for Content Management System (CMS) external communication channels (Chatbot, USSD and Mobile Application), Data extraction forensic workstation for law enforcement and Data Storage Unit and Modems to the Business Regulatory Review Agency (BRRA), Zambia Development Agency (ZDA), Ministry of Green Economy and Environment (MGEE), Competition and Consumer Protection Commission (CCPC), and Patents and Companies Registration Agency (PACRA)

| 1. Item number | 2. Specifications required |  |  | 3. Specifications offered |  | 4. Notes, remarks, ref to documentation |  | 5. Evaluation committee’s notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Laptops |  |  |  |  |  |  |  |  |
|  | Quantiy 77 |  | - Operating system: Windows 11 Pro Processor family: 12th Generation or higher - Intel® Core™ i7 processor Processor: Intel® Core™ i7-12700H Memory: 16 GB DDR4 - Ports: USB Type C/A HDMI - Storage: 512 GB SSD Display: 15.6inch Graphics: Intel® UHD Graphics - Connectivity: Wifi 802.11 ac/5, Ethernet port RJ45 - Backlit keyboard - Laptop Bag and wireless mouse |  |  |  |  |  |  |
| 2 | Desktops |  |  |  |  |  |  |  |  |
|  | Quantity 45 |  | - Operating system: Windows 11 Pro Processor family: 12th Generation Intel® Core™ i7 processor Processor: Intel® Core™ i7-12700 Memory: 16 GB DDR4 Storage: 512 GB SSD - Connectivity: Wifi 802.11 ac Ethernet port RJ45 - Screen Size: 21.5inch - Screen resolution: 1080p |  |  |  |  |  |  |
| 3 | All-in-One Desktop Computers |  |  |  |  |  |  |  |  |
|  | Quantity 28 |  | - Processor Intel Core i7-9700, 3.00 GHz - Screen Size 23.8 inches - Storage (SSD) 256 GB - Operating System Windows 11 Pro- - Type All-in-One PC - Form Factor All-in-One - Graphics Intel UHD Graphics 630 (Integrated) - Connectivity: Wifi 802.11 ac Ethernet port RJ45 - Maximum RAM Capacity 16GB DDR4 |  |  |  |  |  |  |
| 4 | Data Extraction Workstation |  |  |  |  |  |  |  |  |
|  | Quantity 1 | Mobile Data Extraction Workstation- Laptop - Processor: Intel Core i9-13900H or higher - Memory (RAM): 64GB DDR5 or higher - Storage: Primary Drive: 2TB NVMe - Secondary Drive: 2TB NVMe SSD - Graphics Card: NVIDIA RTX 3080 8GB VRAM Display: 15.6-inch 2K resolution |  |  |  |  |  |  |  |  |
|  | Quantity 1 | Data Extraction Workstation- Desktop - Form Factor: Mini- Tower - CPU: Intel Core i9-12900 - Memory: 128GB DDR5 - OS Drive: 1TB NVMe SSD - Processing Drive: 1TB NVMe PCIe Gen 4 SSD - Storage: 4TB SSD - GPU: RTX 3060 12GB memory - OS: Windows 11 Pro |  |  |  |  |  |  |  |  |
| 5 | Computer UPS |  |  |  |  |  |  |  |  |
|  | Quantity 40 |  | - 650VA/400W Floor/Wall Mount - 230V, 8x CEE 7/3 Schuko outlets - USB Type A Port |  |  |  |  |  |  |

| 1. Item number | 2. Specifications required |  | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 6 | Adobe Creative Suite (CS) |  |  |  |  |
|  | Quantity 1 |  |  |  |  |
| 7 | Microsoft Office Enterprise |  |  |  |  |
|  | Quantity 150 | N/A |  |  |  |
| 8 | Microsoft Power bi |  |  |  |  |
|  | Quantity 1 | N/A |  |  |  |
| 9 | Office 2024 (Volume License) |  |  |  |  |
|  | Quantity | N/A |  |  |  |
|  | 1 |  |  |  |  |
| 10 | Windows Server Licences (Windows Server 2025 Datacentre) |  |  |  |  |
|  | Quantity 4 | N/A |  |  |  |
| 11 | Servers for CMS External Communication Channels (Chatbot, USSD and Mobile Application) |  |  |  |  |
|  | Quantity 2 | - Gen11 Server Computer with a 16-core processor, a base clock speed of 2.0 GHz and a maximum turbo frequency of 3.4 GHz, Cache: 30 MB L3 cache, Memory Support: DDR5, up to 4400 MT/s. - GHz, 128 GB (4x32 GB) Dual Rank Memory, one Gen11 x8 Lanes 4GB Cache OCP SPDM Storage Controller, Drive Bay(s) for 8 small form factor drives, one Gen11 1U Standard Fan Kit, two 960GB SATA 6G Mixed Use SFF BC Multi-Vendor SSDs, two 1000W Flex Slot Titanium Hot Plug Power Supply Kits, . |  |  |  |
| 12 | Data Storage Unit |  |  |  |  |
|  | Quantity 1 | - Protocols: SMB 3.0, NFS v3/v4, iSCSI - Compatibility: windows, Linux, App ecosystem with backup, sync, antivirus, and VMs - Connectivity: 10GbE - Form factor: Rack mount - Memory: 32GB ECC RAM - Storage: 4x 6TB - Storage protocol: RAID 1-6 |  |  |  |
|  | Modems |  |  |  |  |
| 13 | Quantity 15 | - LTE 5G/ modem Backup Battery Download Speed Up to 150 Mbps |  |  |  |

Lot 2: Supply, Delivery, Installation and Commissioning Heavy Duty Printers, Office Printers, Specialised Scanners, Projectors and Shredder to the Business   Regulatory Review Agency (BRRA), Zambia Development Agency (ZDA), Ministry of Green Economy and Environment (MGEE), Competition and Consumer Protection Commission (CCPC), and Patents and Companies Registration Agency (PACRA)

| 1. Item number | 2. Specifications required |  |  | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Heavy Duty Printer 4 in 1 |  |  |  |  |  |
|  | Quantity |  |  |  |  |  |
|  | 10 |  |  |  |  |  |
|  | Category |  | Monochrome Multifunctional Specification |  |  |  |
|  | Operation panel |  | - 177.8 mm (7 inch) full-colour touch panel display |  |  |  |
|  | Engine speed |  | - Up to 60 pages A4 per minute |  |  |  |
|  | Duplex print speed |  | - 43 pages per minute, A4 |  |  |  |
|  | Duplex copy speed |  | - 43 pages per minute, A4 |  |  |  |
|  | Resolution |  | - 1200 x 1200 dpi (print); 600 x 600 dpi (scan/copy) |  |  |  |
|  | Recommended Print Volume |  | - 6,000 images per month |  |  |  |
|  | Maximum Print Volume |  | - 300,000 images per month |  |  |  |
|  | Maximum Life |  | - 1,000,000 images or 5 years |  |  |  |
|  | Recommended Scan Volume |  | - 4,800 images per month |  |  |  |
|  | Warm-up time |  | - Approx. 25 seconds or less |  |  |  |
|  | Time to first print |  | - Approx. 4.5 seconds or less |  |  |  |
|  | Time to first copy |  | - Approx. 6 seconds or less (DP); Approx. 7 seconds or less (Platen) |  |  |  |
|  | CPU |  | - ARM Cortex-A53 Dual core 1.4GHz |  |  |  |
|  | Memory |  | - Standard 1.5GB; Max. 3.5GB (optional) |  |  |  |
|  | Standard interfaces |  | - USB 2.0 (Hi-Speed), 1 x USB Host Interface, Gigabit Ethernet, optional Wireless LAN, 1 x eKUIO slot, Slot for optional SD Card |  |  |  |
|  | Integrated accounting |  | - 100 department codes |  |  |  |
|  | Dimensions (W x D x H) |  | - Without packaging: 480 x 495 x 610 mm (without protrusions) With packaging: 633 x 570 x 730 mm |  |  |  |
|  | Weight |  | - Without packaging: Approx. 25.2 kg (with bundled toner) With packaging: 32 kg |  |  |  |
|  | Power source |  | - AC 220 ~ 240 V, 50/60 Hz |  |  |  |
|  | Power consumption |  | - Printing: 740.8 W Copying: 742 W Ready mode: 57.9 W Low power mode: 38 W Sleep-mode: 0.5 W |  |  |  |
|  | Noise level |  | - Copying/Printing: 58 dB (A) Stand-by: 33 dB (A) |  |  |  |
|  | Safety standards |  | - TÜV/GS, CE, ISO 9001, ISO 14001 |  |  |  |
|  | Paper Handling |  |  |  |  |  |
|  | Input capacity |  | - Multi-purpose tray: 100 sheets, 60 to 220 g/m² Standard universal paper cassette: 500 sheets, 60–120 g/m² |  |  |  |
|  | Max. input capacity |  | - 2,600 sheets with options |  |  |  |
|  | Duplex |  | - Standard duplex unit, 60 to 120 g/m² |  |  |  |
|  | Document Processor |  | - Dual scan document processor, 100 sheets, 50–120 g/m² |  |  |  |
|  | Output capacity |  | - Max. 500 sheets face-down with paper full sensor |  |  |  |
|  | Print Functions |  |  |  |  |  |
|  | Controller language |  | - PRESCRIBE |  |  |  |
|  | Emulations |  | - PCL6, KPDL3, PDF Direct Print, XPS Direct Print, AES support, Open XPS, URF, PWG Raster, PCLm, Line Printer, IBM Proprinter, Epson LQ-850 |  |  |  |
|  | Operating systems |  | - Windows, MAC OS X, Chrome OS, Linux |  |  |  |
|  | Fonts/barcodes |  | - 93 outline fonts, 4 Calibri fonts, 4 Cambria fonts, 1 Bitmap font, 45 one-dimensional barcodes, 1 two-dimensional barcode (PDF417) |  |  |  |
|  | Print features |  | - PDF Direct Print, Secure Printing, Encrypted PDF Direct Print, TLS 1.3, Secure Boot, Universal Print |  |  |  |
|  | Mobile printing support |  | - Mobile Print app for iOS and Android, AirPrint, Mopria, Microsoft Universal Print, Wi-Fi Direct (Opt) |  |  |  |
|  | Copy Functions |  |  |  |  |  |
|  | Max. original size |  | - A4/Legal |  |  |  |
|  | Continuous copying |  | - 1–999 |  |  |  |
|  | Zoom range |  | - 25–400% in 1% steps |  |  |  |
|  | Preset magnification |  | - 7 Reductions / 5 Enlargements |  |  |  |
|  | Image adjustments |  | - Text + Photo, Photo, Text, Chart/Map, Printed document |  |  |  |
|  | Features |  | - Electronic sort (collate), ID card copy, 2in1, 4in1, Auto cassette change, Skip blank page, Skip Line, DP Auto-cropping |  |  |  |
|  | Scan Functions |  |  |  |  |  |
|  | Functionality |  | - Scan-to-Email, Scan-to-FTP, Scan-to-SMB, Scan to USB Host, Network TWAIN, WIA, WSD scan, Chrome OS (Scan), AirPrint, Mopria |  |  |  |
|  | Scan speed Simplex |  | - 60 ipm (300 dpi, A4, b/w); 40 ipm (300 dpi, A4 color) |  |  |  |
|  | Scan speed Duplex |  | - 120 ipm (300dpi, A4, b/w); 80 ipm (300 dpi, A4, color) |  |  |  |
|  | Max. scan size |  | - A4, Legal, Banner paper (max 915mm) |  |  |  |
|  | File types |  | - TIFF, JPEG, XPS, Open XPS, PDF, PDF/A, Encrypted PDF, High compression PDF |  |  |  |
|  | Fax Functions |  |  |  |  |  |
|  | Compatibility |  | - Super G3 |  |  |  |
|  | Modern speed |  | - Max. 33.6 kbps |  |  |  |
|  | Transmission speed |  | - Max. 3 seconds (JBIG) |  |  |  |
|  | Address book |  | - 200 entries |  |  |  |
|  | Scanning density |  | - Normal: 200 x 100 dpi Fine: 200 x 200 dpi Superfine: 200 x 400 dpi Ultrafine: 400 x 400 dpi |  |  |  |
|  | Maximum original size |  | - A4/Legal, Banner (max 915mm) |  |  |  |
|  | Features |  | - Network fax, Duplex transmission/reception, Encrypted transmission/reception, Polling transmission/reception, Broadcast |  |  |  |
|  | Consumables |  |  |  |  |  |
|  | Toner Kit |  | - Microfine toner black for 40,000 pages A4 |  |  |  |
|  | Staple cartridge |  | - 2 x 1,500 staples |  |  |  |
|  | Options |  |  |  |  |  |
|  | Paper feeder |  | - Max. 500 sheets, 60 - 120 g/m² |  |  |  |
|  | Paper tray |  | - 250-sheet face-up output tray |  |  |  |
|  | Stapler |  | - Up to 20 sheets one position semi-automatic stapling |  |  |  |
|  | Cabinet |  | - Wooden desk with storage capacity |  |  |  |
|  | Caster Unit |  | - For increased stability |  |  |  |
|  | Memory upgrade |  | - 1GB/2GB, DDR3-144pin DIMM |  |  |  |
|  | Storage Devices |  | - SSD hard disk: 64GB/256GB/512GB |  |  |  |
|  | Security Features |  | - TPM (Trusted Platform Module), USB IC Card Reader + Card Authentication Kit, Network Encryption |  |  |  |
|  | Warranty |  | - Standard Warranty-1-year warranty |  |  |  |
| 2 | Office Printer-Leserjet |  |  |  |  |  |
|  | Quantity 19 |  | - Medium All in one printer - Monochrome Laser Multifunctional - Print, Copy, Scan, Send - NETWORK Standard: 1000Base-T/100Base-TX/10-Base-T, Wireless LAN (IEEE 802.11 b/g/n), Wi-Fi Direct Connection - 1-sided: Up to 43/43 ppm (A4) 2-sided: Up to 36/36 ppm (A4) OTHERS Standard: USB 2.0 (Host) x2, USB 2.0 (Device) x1 |  |  |  |
| 3 | Specialized Scanners for Digitization of Business Records |  |  |  |  |  |
|  | Quantity |  |  |  |  |  |
|  | 10 |  |  |  |  |  |
|  | Office Document Scanner Specifications |  |  |  |  |  |
|  | Type |  | - Office Document Scanner |  |  |  |
|  | Document Feeding |  | - Automatic or Manual |  |  |  |
|  | Document Size |  | - Width: 50 - 216 mm (1.97” – 8.5”) |  |  |  |
|  |  |  | - Length: 54 - 356 mm (2.13” – 14.02”) |  |  |  |
|  | Long Document Mode |  | - Up to 5,588 mm (220”) |  |  |  |
|  | Document Weight |  | - 80 g/m² (7 lb. – 110 lb. Bond Plain Paper) |  |  |  |
|  | Feeder Capacity |  | - Up to 80 Sheets |  |  |  |
|  | Grayscale |  | - 8-bit |  |  |  |
|  | Color |  | - 24-bit |  |  |  |
|  | Light Source |  | - RGB LED |  |  |  |
|  | Scanning Element |  | - Contact Image Sensor (CMOS) |  |  |  |
|  | Scanning Modes |  | - Black & White, Error Diffusion, 256-level Grayscale, Advanced Text Enhancement I & II, 24-bit Color |  |  |  |
|  | Optical Resolution |  | - Up to 600 dpi |  |  |  |
|  | Output Resolution |  | - 100/150/200/240/300/400/600 dpi |  |  |  |
|  | Max. Scanning Speeds |  | - Simplex: Up to 60 ppm, Duplex: Up to 120 ipm (Black & White, Grayscale, Color) |  |  |  |
|  | Interface |  | - USB 3.1 Gen 1 |  |  |  |
|  | Dimensions (H x W x D) |  | - 231 x 285 x 254 mm (9.09” x 11.22” x 10”) (with trays closed) |  |  |  |
|  | Weight |  | - Approx. 3.4 kg (7.5 lbs) |  |  |  |
|  | Power Consumption |  | - 23W or Less (Sleep Mode: 1.4W or less) |  |  |  |
|  | Suggested Daily Volume |  | - Up to 7,500 scans |  |  |  |
|  | Supported Operating Systems |  | - Windows 7 (SP1 or later), Windows 8.1, Windows 10 (32-bit/64-bit), Windows Server 2008 R2 or newer |  |  |  |
|  | Additional Features |  | - Automatic Color Detection, Page Size Detection, Auto Resolution, Background Smoothing, Character Emphasis, Color Dropout, Deskew, Double-Feed Release, Feeding Retry, Image Rotation, Long Document Mode, MultiStream™, Pre-Scan, Prevent Bleed-Through, Shadow Cropping, Skip Blank Page, Text Enhancement, Text Orientation Recognition, Ultrasonic Double-Feed Detection, User Preferences |  |  |  |
|  | Optional Accessories |  | - Flatbed Scanner Unit, Barcode Module, 2D Code Module, Exchange Roller Kit, Passport Carrier Sheet, A4/LTR Carrier Sheet |  |  |  |
| 4 | Projectors |  |  |  |  |  |
|  | Quantity 2 |  | - Outdoor Portable Projector - 1000 ANSI Lumens - Display Resolution 1920 x 1080 - AI-Powered Image - Built-in Battery with 5 Hour Playtime - IPX3 Water Resistance, Drop Resistant, Dust Resistance |  |  |  |
| 5 | Shredder |  |  |  |  |  |
|  | Quantity 1 | - 100 sheets - Sheet Capacity 18 / 20 - Shred Size: 1/8" x 1 9/16" - Feed Width (in) 10.25" - Media Types: paper, Staples & Paper Clips, Credit Cards, CDs / DVDs |  |  |  |  |

Lot 3:  Supply, Delivery, Installation and Commissioning of an All-in-One Video Conferencing Kit and Digital Camera to the Patents and Companies Registration Agency (PACRA) and Zambia Development Agency (ZDA)

| 1. Item number | 2. Specifications required |  | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 1 | All-in-One Video Conferencing Kit-PACRA |  |  |  |  |
|  | Quantity 1 | - Display Screen​ Minimum 86-inch and Above 4K UHD interactive display with multi-touch capability for enhanced collaboration. Anti-glare coating for clear visibility in various lighting conditions. - Built-in PC (OPS) Core i7 processor with at least 16GB RAM and 512GB SSD. Supports Windows 10/11 or Linux for seamless video conferencing applications. - Video Conferencing System High-definition (HD) video conferencing system with AI-enhanced camera tracking, noise suppression, and echo cancellation. Includes multiple microphones for clear audio capture in large meeting rooms. - Camera PTZ (Pan-Tilt-Zoom) camera with 4K resolution, wide-angle lens, and auto-framing for active speaker tracking. - Speakers & Audio Advanced speaker system with deep bass and crisp highs. Omnidirectional microphones with noise cancellation technology for crystal-clear communication. - Connectivity HDMI, USB-C, USB 3.0, RJ-45 Ethernet, Wi-Fi, and Bluetooth support for flexible integration with multiple devices. - Mounting Options Wall-mount bracket for fixed installation. Mobile monitor stand for flexible placement in different meeting spaces. - Compatibility Compatible with major video conferencing platforms such as Zoom, Microsoft Teams, Google Meet, Cisco Webex, and Skype for Business. |  |  |  |

| 1. Item number | 2. Specifications required |  | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 2 | Digital Camera |  |  |  |  |
|  | Quantity 1 | - Telephoto Zoom Lens - 5D MIV - 30MP - Full frame CMOS Sensor - ISO 100 – 32000 (expands to 50 - 102400) - 3.20" Fixed Type Screen - Optical (pentaprism) viewfinder - 7.0fps continuous shooting - 4K at 30fps, FHD at 60fps and HD at 120fps Video Recording - Built-in Wireless - Built-in GPS - 890g. 151 x 116 x 76 mm - Weather-sealed Body |  |  |  |

Lot 4:  Supply, Delivery, Installation and Commissioning of Data Extraction Forensic Toolkit for Law Enforcement to Competition and Consumer Protection Commission (CCPC)

| 1. Item number | 2. Specifications required |  | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Data Extraction Forensic Toolkit for Law Enforcement |  |  |  |  |
|  | Quantity 1 1 1 1 1 | Composed of; |  |  |  |
|  |  | Data Write Blockers |  |  |  |
|  |  | - Forensic Write-Blocking Drive Dock - Host (I/O) Ports: (1) USB 3.2 Gen 2 Type-C: up to 10 Gbps (Actual transfer speeds up to 6 Gbps) - Drive Types Support: ATA/IDE 2.5" drives PATA/IDE 3.5 SATA 2.5" drives SATA 3.5" drives SATA SSD (solid state drives) Hitachi 1.8" drives Toshiba 1.8" drives MacBook Air 2010 MSATA Mini PCIe PATA Mini PCIe SATA Mini PCIe USB - Sector Size Compatibility: Compatible with 512, 512e and 4Kn drive types - Supports S.M.A.R.T. disk info: Yes - Operating system compatibility:Windows 10 and 11 - macOS 11 “Big Sur” or newer - Red Hat Enterprise Linux and Ubuntu LTS - Allows HPA/DCO override: Yes, user-configurable options - Construction material(s): Rugged aluminum construction |  |  |  |
|  |  | USB Write Blocker |  |  |  |
|  |  | NVMe WriteBlocker |  |  |  |
|  |  | Data Extraction Software - Magnet AXIOM Forensics |  |  |  |
|  |  | Mobile Edit |  |  |  |